Shoroog almalki

Artificial Intelligence in Robotics

Task :

Use Slam approach with TurtleBot3

To do simulation and mapping you should follow these steps :

- Open a terminal window type "source devel/setup.bash"  then write this command :

$ export TURTLEBOT3_MODEL=burger

There are three model of turtlebot3 (burger,waffle or waffle_pi) , we can chose any one of them ,in my case it is burger . then type this command to launch gazebo :

$roslaunch turtlebot3_gazebo turtlebot3_world.launch .

- Open new terminal  window , type "source devel/setup.bash"  then write this command :

$ export TURTLEBOT3_MODEL=burger . then type this command to launch SLAM :

$ roslaunch turtlebot3_slam turtlebot3_slam.launch slam_methods:=gmapping .

- To remotely control and move the robot , open new terminal  window , type "source devel/setup.bash"  then write this command :

$ export TURTLEBOT3_MODEL=burger .

Then type :

$ roslaunch turtlebot3_teleop turtlebot3_teleop_key.launch

After moving the turtlebot3 in all direction you should have image .

4 – after that you can save the map by typing this command :

$ rosrun map_server map_saver -f ~/map

Then you will get "map.pgm"

The output